Miami Tower and Boca Raton FCT

Letter of Agreement

EFFECTIVE:  August 1, 2011

SUBJECT:  Control of IFR/SVFR Aircraft

1.	PURPOSE.  This Letter of Agreement (LOA) defines the responsibilities of Miami ATCT (MIA) and Boca Raton FCT (BCT) and is supplemental to FAA Order 7110.65 and FAA Order 7210.3.

2.	CANCELLATION. Miami Tower and Boca Raton FCT Letter of Agreement, subj: Control of IFR/SVFR Aircraft, dated October 1, 2007.

3.	RESPONSIBILITIES.

a.	General

(1)	BCT must notify MIA of opening/closing.

(2)	BCT must forward "runway in use" to MIA via landline.

(3)	BCT must notify MIA via landline when weather changes to/from visual approach minimums and IFR conditions.

b.	Departures

(1)    BCT tower must amend departures with MIA area destinations to the following altitudes and issue the routing as provided:

| TYPE | ALTITUDE | ROUTING |
| --- | --- | --- |
| PROPS | 30 | RV DESTINATION ARPT |
| TURBOPROPS | 70 | RV V295 VKZ DIRECT DESTINATION ARPT |
| TURBOJETS | 70 | RV V295 VKZ DIRECT DESTINATION ARPT |

(2) After obtaining a release from PBI, BCT must coordinate and receive approval for Runway 23 IFR departures with MIA and provide Miami approach with the aircraft beacon code.

c.	Arrivals

(1)	MIA shall forward to BCT arrival information on all IFR, and VFR aircraft conducting a practice instrument approach, prior to 10 flying miles from the airport and transfer communications on all arrivals not later than 5 NM from the airport.

(2)	BCT must notify MIA of a RWY 23 IFR missed approach as soon as feasible.  RWY 23 "turn right heading 340, maintain 2,000.

(3)	VFR practice approach aircraft that request follow-on radar service from MIA must be routed by BCT to the west at or below 1,600.

___________________________		___________________________

William W. Kribble					David L. Webster

South Florida District Manager			Air Traffic Manager,	Boca Raton FCT